ThRACE Mission Statement Thermal and Radiation Analysis CubeSat Experiment

| Document | Revision | Status | Date |
| --- | --- | --- | --- |
| TRACE-MS-001 | Rev 1.0 | DRAFT | 2026 |

1. MISSION OVERVIEW

ThRACE is a 2U CubeSat fully designed and built by a high school student. Being a technological demonstration, it has the task of characterizing the radiation and thermal environment in Low Earth Orbit (LEO). ThRACE does this by measuring particle radiation flux, surface temperature on every face of the satellite, as well as how the strength of Earth’s magnetic field changes throughout orbits. Every piece of data is georeferenced in order to validate simulated data, verifying that the shielding and thermal control design on the satellite was correct when compared to actual data.

This mission is made almost entirely from scratch. Using a very limited amount of commercial components, it is meant to show what is possible. These custom made parts include but are not limited to the OBC, power system, ground station, and tracking system. Commercial hardware is used when it would make the most sense. ThRACE, like mentioned earlier, serves both as a scientific instrument and a demonstration of engineering ability.

2. MISSION OBJECTIVES

- OBJ-1: Measure radiation dose rate as a function of orbital position using a Geiger-Müller tube, producing a georeferenced radiation map along the ground track.

- OBJ-2: Characterize the thermal environment of a 2U CubeSat across sunlight and eclipse cycles by logging surface temperatures from PT1000 RTD sensors on each face.

- OBJ-3: Map the local geomagnetic field along the orbital path using a three-axis magnetometer and compare measured values to the International Geomagnetic Reference Field (IGRF) model.

- OBJ-4: Validate the satellite's passive radiation shielding design (2mm Al6061 + 1mm polyethylene liner) by comparing measured dose rates to pre-flight model predictions.

- OBJ-5: Validate the satellite's thermal control design (internal MLI blankets, bottom-face radiator) by comparing measured temperature cycles to thermal simulation predictions.

- OBJ-6: Demonstrate reliable operation of a custom-designed OBC and custom EPS in space for the entire duration of the mission.

3. ORBIT AND MISSION PARAMETERS

| Parameter | Value |
| --- | --- |
| Form Factor | 2U CubeSat (10 × 10 × 20 cm) |
| Target Orbit | Low Earth Orbit (LEO) |
| Target Altitude | ~500 km |
| Inclination | 45° |
| Orbital Period | ~95 minutes |
| Eclipse Fraction | ~37% per orbit (~35 min eclipse / ~60 min sunlight) |
| Target Mission Duration | 12-24 months |
| Estimated Mass | ~1.8 – 2.4 kg (estimated) |
| Avg Power Generation | ~2.5 – 4W |
| Launch Vehicle | TBD |

4. SPACECRAFT OVERVIEW

ThRACE is built around a 2mm Al6061 aluminum chassis with a 1mm high-density polyethylene inner liner providing passive radiation shielding. Internal MLI blankets provide thermal insulation, and the bottom face of the satellite is left uninsulated to serve as a dedicated radiator surface. The satellite is designed to be compliant with the CubeSat Design Specification (CDS) Rev 14.

| Subsystem | Approach | Implementation |
| --- | --- | --- |
| OBC | Custom | Cortex-M7, FreeRTOS, custom KiCad PCB, CCSDS telemetry |
| EPS | Custom | SPV1040 MPPT, BQ25703A BMS, regulated 3.3V / 5V rails, custom KiCad PCB |
| ADCS | COTS | CubeSpace CubeADCS — reaction wheels + magnetorquers, sun-pointing mode |
| COMMS | COTS | Endurosat UHF Type II transceiver, 9.6 kbps AX.25 / CSP, deployable turnstile |
| Structure | COTS + Custom | Commercial 2U Al6061 chassis + custom PE liner, MLI, deployable solar wings |
| Flight SW | Custom | FreeRTOS task architecture, cFS-inspired design, autonomous FDIR, store-and-forward |
| Ground Station | Custom | DIY SatNOGS rotator, Arrow Antenna II, Nooelec LNA, RTL-SDR V4, Direwolf, InfluxDB, Grafana, Cesium 3D visualization |

5. SCIENCE PAYLOAD

| Sensor | Measurement | Science Value |
| --- | --- | --- |
| Geiger-Müller Tube (SBM-20 or J305) | Particle radiation flux (CPM, µSv/hr) | Maps radiation dose rate vs. orbital position. Identifies South Atlantic Anomaly crossing events. Validates passive shielding predictions. |
| PT1000 RTD × 6 (one per face) | Surface temperature (°C, all faces) | Characterizes the thermal environment across a full orbital cycle. Validates MLI blanket and radiator face thermal model predictions. |
| 3-Axis Magnetometer (via CubeADCS) | Magnetic field vector (nT) | Maps local geomagnetic field along ground track compared to the IGRF model. Secondary use: magnetorquer desaturation reference. |
| Solar Irradiance Sensor | Solar flux (W/m²) | Validates solar panel power generation model. Correlates eclipse transitions with thermal and power system data. |

6. MISSION SUCCESS CRITERIA

Minimum Success

- Satellite survives launch and deploys from P-POD successfully

- UHF beacon received by ground station or SatNOGS network within first 3 orbits

- At least one full telemetry downlink session completed with all sensor readings present

- Satellite operates for a minimum of 30 days without unrecoverable failure

Full Mission Success

- All six mission objectives (OBJ-1 through OBJ-6) met with continuous data collection for 12+ months

- Radiation map covering 100+ orbits with South Atlantic Anomaly clearly identifiable in dataset

- Measured face temperatures match thermal simulation predictions within ±10°C RMS

- Measured radiation dose within ±25% of shielding model prediction

- Dataset published with DOI and shared with the SatNOGS and amateur satellite communities

- Real-time mission data publicly accessible via Grafana dashboard and Cesium 3D visualization

7. HIGH-LEVEL MISSION TIMELINE

| Phase | Description |
| --- | --- |
| M1–6 | Education, ground station build, system design and budgets, KiCad migration |
| M4–9 | Custom OBC and EPS PCB design, fabrication, and bring-up. Flight software development. |
| M9–12 | Payload and support board fabrication. Chassis build. Solar panel and deployable wing assembly. |
| M12–17 | Full system FlatSat test (including 48-hour soak test). Chassis integration. Environmental testing. |
| M16–20 | Portfolio documentation, GitHub publication, college applications, faculty outreach. |
| M18–36+ | University affiliation, NASA CSLI application, FCC licensing, flight unit upgrades, TVAC/vibration testing, launch campaign. |

8. DOCUMENT INFORMATION

| Author | Cortland Familian |
| --- | --- |
| Mission | ThRACE — Thermal and Radiation Analysis CubeSat Experiment |
| Document ID | ThRACE-MS-001 |
| Revision | Rev 1.0 — Initial Draft |

This document is a living reference and will be updated as design decisions are finalized. Refer to the version number in the header to confirm currency.